Networking
Hardware requirements.

1) ethernet card, NIC(network interface card), LAN card.
2)cabals
3)Network device

logical Terms:
IP address
mac address
source and destination address

IPv4(internet protocol)  - 32 bit


IP class

A class- 0-126
B class   128-191
C class 192-223
D class 224-239
E class 240-255

127 ip is reserved for loopback or localhost.

Class D and class E is for special purpose or research purpose



Network address: is the first IP address of any IP range.
Broadcast address: this is the last IP address of any IP range.

this class have their networks and host addresses
class A -   N.H.H.H
class B -    N.N.H.H
class C -    N.N.N.H

Suppose my IP is 192.168.1.60 and I want to calculate my network address and broadcast address

this is from class C contain 3 Network beat and 1 host
then simple 192.168.1.0  this is network address
and just convert to 0-255 make broadcast address = 192.168.1.255

and remain are IP address
192.168.1.1  ----  192.168.1.254
the number of usable IP address in class c in 1 network are total 256-2 =254 IP address.

EX-2)  172.20.1.215
class B N.N.H.H
Network address  172.20.0.0
broadcast address 172.20.255.255

and remain are usable IP address in single network
172.20.0.1 ---- 172.20.0.255 ---172.20.1.0---172.20.1.255---172.20.2.0----172.20.2.255
and last IP is 172.20.255.254   cause this 172.20.255.255 is broadcast

IP addresses=  256*256-2=65536-2=65534


Ex-3) 56.104.25.99
Class A   N.H.H.H
network Address 56.0.0.0
broadcast address 56.255.255.255

and remain are usable IP address in single network
59.0.0.1-----59.0.0.255---59.0.1.0---59.0.1.255---59.0.2.0---59.0.2.255 and more
so last IP is 59.255.255.254

IP addresses = 256*256*256-2=16777214

subnetting:
suppose we have 5 different departments IT,HR,Finance and other they all in same network that is 192.168.1.0 so they can communicate with each other but we don’t want it in this network there are total 254 IP addresses usable and we need to give all departments 25 each and they don’t isolated from each other so we use subnetting for this.
                                                               or
subnet = sub network,   means devide the network into multiple logical sub network. 
subnet mask= A subnet mask is a 32-bit number that divides an IP address into a network and host portion

subnet class A = 255.0.0.0

Class B = 255.255.0.0
class C = 255.255.255.0

if you give plain IP so we can check from which class by noticing their first octet
and it IP have subnet then this IP is belongs to their class
like 192.168.1.45   this is class C, but we apply the subnet on this
like 192.168.1.45/255.255.0.0   then this IP is from class B
like 192.168.1.45/255.0.0.0  then this IP is from class A

another way to wright subnet mask (CIDR)
Class A subnet =255.0.0.0   it contain 8 bit on so /8
class B subnet = 255.255.0.0 it contain 16 bit on so /16
class C subnet = 255.255.255.0 it contain 24 bit on so /24

192.168.1.45/8   class A
192.168.1.45/16  class B
192.168.1.45/24   class C


video 2                                                         mobile   private IP


 DHCP           public IP                                                    laptop    private IP
ISP ---------router                                                             PC        private IP

(internet service provider)

when server makes request youtube.com then it first goes to the router then source address of server is changed means source IP from private to public using the NAT(Network Address Translation). The youtube server get our public IP of router not an private of our server then again from reverting back request it reach to router then it change destination address from router public IP to server private IP and then we get the output of youtube.

| Feature | Private IP Address | Public IP Address |
| --- | --- | --- |
| Definition | IP address used within a private network (LAN) | IP address used to identify devices on the Internet |
| Accessibility | Only accessible within internal networks | Accessible globally over the Internet |
| Example Ranges | - 10.0.0.0 – 10.255.255.255 - 172.16.0.0 – 172.31.255.255 - 192.168.0.0 – 192.168.255.255 | Any IP not in private range (e.g. 8.8.8.8, 1.1.1.1) |
| Assigned By | Chosen by network admin or DHCP | Assigned by ISP or IANA |
| Used For | Internal communication (e.g., home, office LAN) | Communication over the internet |

ISP provides and internet to router then router get an public IP and this given from DHCP server from ISP. Then all devices which is connected to router get an private IP address.

and is there any request from mobile to internet it goes like mobile  routerinternet server   and the IP get this server is not mobile private IP it gets router Public IP.
public IP ==Real IP
Private IP== virtual IP


server-client model
server =  who serve the services
client  =  who use this services

Total ports in system = 65535
socket=ip:port
client   webserver
when client send request to server it get IP only but application not works on it ,the application works on ip:ports


dig: (domain information grouper)
it is same like nslookup but it is advanced.

| Feature | dig | nslookup |
| --- | --- | --- |
| Output format | Structured, detailed | Simpler, less structured |
| Scripting usage | Easier (with +short, etc.) | Harder to parse in scripts |
| Standards | Modern, follows DNS RFCs closely | Slightly outdated behavior |
| Popularity | Preferred by professionals | Still available but less favored |
| Development | Actively used in diagnostics | Considered deprecated by some |

| Feature | IPv4 | IPv6 |
| --- | --- | --- |
| Full Name | Internet Protocol version 4 | Internet Protocol version 6 |
| Address Size | 32 bits | 128 bits |
| Address Format | Decimal, dotted format | Hexadecimal, colon-separated |
| Example Address | 192.168.0.1 | 2001:0db8:85a3:0000:0000:8a2e:0370:7334 |
| Configuration | Manual or via DHCP | Auto-config or DHCPv6 |

network devices:
Hub
Switch
Router

1.Hub
in this Hub case: 4 host connected to Hub when host 1 want to connect to host3 then host 1 make request to host 3 then first it goes to Hub then Hub broadcast all host who is host3 (IP) then Host3 say I am then Host1 connect to host3.
but one issue in Hub if host2 says I am host3 then Hub directly connect to host2 without any checking
host1   HUB     host2

2.switch 
same like Hub the switch firstly broadcast the each host and one thing he captured the mac of each host and stored in 	mac table and in this table contain data like IP=Mac address.
and next time any host perform request it not broadcast this time he did unicast directly connect to right host based on this mac table.

 3.Router
communication between different networks.
and it provides route to our network.

TCP/IP model:

Contain 4 layers
1. Application
2. transport layer   (TCP/UDP) TCP=transmission control protocol  UDP=user datagram protocol
3. network layer
4. physical or datalink layer

| Feature | TCP (Transmission Control Protocol) | UDP (User Datagram Protocol) |
| --- | --- | --- |
| Connection | Connection-oriented (handshake before data) | Connectionless (no handshake) |
| Reliability | Reliable – ensures data is delivered in order | Not reliable – no guarantee of delivery |
| Speed | Slower (due to checks and acknowledgments) | Faster (no checks or confirmation) |
| Use Case | File transfer, web browsing, emails (when data must arrive correctly) | Streaming, gaming, voice calls (speed > accuracy) |
| Error Checking | Yes, with acknowledgment and retransmission | Minimal error checking |
| Examples (Ports) | HTTP (80), HTTPS (443), FTP (21), SSH (22) | DNS (53), DHCP (67/68), VoIP (5060) |

video 3

there are types of ethernet card 
1)ethN == eth1, eth2, eth3
2)enoN  == en01, eno2, eno3 (ethernet on)
3)ensN  == ens1, ens2, ens3   
how to work ping command
when we did ping 192.168.1.54 
then my host send echo request(ICMP packets) to this ip server and revert back to the host then connectivity with this host and IP is happened.

when we did ping google.com
it first goes to /etc/hosts if it contain domain with IP address then it revert back to them if not it goes request to /etc/resolve.cong where we mentioned the dns server like 8.8.8.8 or 8.8.4.4 then it goes to the internet.

find IP from domain
use nslookup command

find which port on running
netstat -tulnp
ss -tulnp

connection= collection of setting.
nmcli device status
nmcli connection show

setting of manual IP setup through UI
nmtui
hostnamectl