Networking 

Hardware Requirements

- Ethernet Card / NIC (Network Interface Card) / LAN Card – Enables network connectivity for a system.

- Cables – Physical medium for transmitting data.

- Network Devices – Such as hubs, switches, and routers.

Logical Terms

- IP Address – A unique numerical label assigned to each device in a network.

- MAC Address – A unique hardware identifier assigned to each NIC.

- Source and Destination Address – Used for identifying sender and receiver in communication.

IPv4 (Internet Protocol)

IPv4 is a 32-bit addressing scheme.

IP Classes

- Class A: 0 – 126

- Class B: 128 – 191

- Class C: 192 – 223

- Class D: 224 – 239 (Multicast – special purpose)

- Class E: 240 – 255 (Reserved for research and experimental use)

Class Format

- Class A: N.H.H.H

- Class B: N.N.H.H

- Class C: N.N.N.H

Where N = Network bits, H = Host bits.

Examples

Example 1: 192.168.1.60

- Belongs to Class C.

- Network Address: 192.168.1.0

- Broadcast Address: 192.168.1.255

- Usable Host Range: 192.168.1.1 – 192.168.1.254

- Total Usable IPs: 254

Example 2: 172.20.1.215

- Belongs to Class B.

- Network Address: 172.20.0.0

- Broadcast Address: 172.20.255.255

- Usable Host Range: 172.20.0.1 – 172.20.255.254

- Total Usable IPs: 256 × 256 – 2 = 65,534

Example 3: 56.104.25.99

- Belongs to Class A.

- Network Address: 56.0.0.0

- Broadcast Address: 56.255.255.255

- Usable Host Range: 56.0.0.1 – 56.255.255.254

- Total Usable IPs: 256 × 256 × 256 – 2 = 16,777,214

Subnetting

Subnetting divides a large network into smaller sub-networks for better management and security.

Default Subnet Masks

- Class A: 255.0.0.0

- Class B: 255.255.0.0

- Class C: 255.255.255.0

CIDR Notation

- Class A: /8

- Class B: /16

- Class C: /24

Example

- 192.168.1.45/8 → Class A

- 192.168.1.45/16 → Class B

- 192.168.1.45/24 → Class C

DHCP, Public IP, and Private IP

- ISP (Internet Service Provider) assigns a Public IP to the router via DHCP.

- Devices (mobile, laptop, PC) connected to the router get Private IPs.

- When accessing the internet, devices use the router’s Public IP (not their private IP).

Public IP = Real IP
Private IP = Virtual/Internal IP

Server-Client Model

- Server: Provides services.

- Client: Consumes services.

- Total Ports Available: 65,535

- Socket: Combination of IP and Port (IP:Port).

Example:
A web client sends a request to a server → The application identifies the correct service using port numbers.

Network Devices

1. Hub

- Broadcasts incoming data to all connected hosts.

- Drawback: Cannot verify the destination host. If a wrong host claims the request, the hub forwards it incorrectly.

2. Switch

- Similar to a hub but maintains a MAC address table.

- Ensures data is sent directly to the correct host after the first broadcast.

3. Router

- Connects different networks.

- Provides routing functionality for data packets.

Ethernet Interface Types

- ethN → e.g., eth0, eth1

- enoN → e.g., eno1, eno2 (Ethernet Onboard)

- ensN → e.g., ens1, ens2

Ping Command

- ping <IP>: Sends ICMP Echo Request packets to test connectivity.

- ping google.com:

  - Checks /etc/hosts for domain resolution.

  - If not found, queries DNS servers defined in /etc/resolv.conf (e.g., 8.8.8.8).

Useful Commands

- Find IP from Domain: nslookup <domain>

- Find Running Ports:

  - netstat -tulnp

  - ss -tulnp

- Network Manager:

  - Show device status: nmcli device status

  - Show connections: nmcli connection show

- Manual IP Setup (UI): nmtui

- Set Hostname: hostnamectl